Form B-100 (section 1)

CONSENT PROCESSING SUMMARY SHEET

PROJECT DETAILS

Project address: 34 CARTERS ROAD

BC number: ABA10121857

Project description: DWELLING ALTERATION: PARTIAL UNDERPINNING PERIMETER FOUNDATIONS

Building category: R1 R2 R3 C1 C2 C3

Processing officer (name and level): Millie Blay Res 1

Date processing started: 18.01.2013

| POST ACCEPTANCE CHECK | Y, Q, RES | Comments, dates, etc: |
| --- | --- | --- |
| Has the building consent application been inputted into GEMS as per instructions from the acceptance vetting tab? Correct project description and building category, the requested PIM/Development Report actions have been started and the requested specialist review actions have been made active. |  |  |
| State classified use of building as defined in Clause A1 Maybe more than one. Has implications for ‘limits on application’ for some Building Code clauses. |  | Housing - detached dwelling |
| Does a copy of the application for building consent have to be forwarded to NZ Fire Service Commission? See page 9 on application form, GD036 and s46, BA 2004. | NA |  |
| Indicate specialist reviews required and ensure actions are made active in GEMS: |  | Specialist reviewers name and date completed: |
| Structure (B1 and B2 Producer Statements) |  |  |
| Fire (C1-4) |  |  |
| Civil (E1) |  |  |
| Producers statements (G4-HVAC, D2-Lifts, G9-Electrical) |  |  |
| Specialist water supplies and trade waste (G12, G13, G14) |  |  |
| Contamination of site (F1) |  |  |
| Hazardous substances (F3) |  |  |
| Food premises (G3) |  |  |
| Sound (G6) |  |  |
| Plumbing and drainage (G12, G13) |  |  |
| Other (please list): |  |  |

| PROCESSING NOTES: | Y, Q, RES | Comments, dates, etc: |
| --- | --- | --- |
| Record any warnings, comments and internal instructions. |  |  |
| Record autotext numbers for conditions/advice notes to be added to B-411 form. |  | 3, 2, 4,49, 89, 12 |
| Record dates of any RFI letters sent, responses received and other communication with applicant. Use B-521 section 1 to send RFI letter to applicant. |  |  |
| Record details of any peer review/supervision by competent officer where building category is above processing officer’s competency level. Where applicable, provide competent officer’s name and date of ‘sign-off’. |  |  |

| EXTERNAL CONTRACTOR CONSENT APPROVAL/REFUSAL: |  |  |
| --- | --- | --- |
| Approval: All relevant technical checks have been made and legislative compliance has been demonstrated on reasonable grounds as per section 49 of the Building Act 2004. The reasons for decisions have been recorded and the consent is approved. |  |  |
| Refusal: All relevant technical checks have been made, legislative compliance has not been demonstrated and it is recommended that the consent is refused as per section 50 of the Building Act 2004. The reasons for refusing the consent have been recorded on the processing checksheet. |  |  |
| Processing Officer: | Date: |  |

| FINAL PROCEDURES CHECKLIST: |  |
| --- | --- |
| All specialist reviews completed, RFI responses checked and consent documents updated and stamped? |  |
| Where a Project Information Memorandum has been requested has this been completed and issued or will be attached? |  |
| Where a development contribution is payable has a Section 36 notice been attached to the PIM? Where no PIM has been requested it is to be attached to the building consent. Use warning note in GEMS. |  |
| Where resource consent approval is still needed has a Section 37 certificate been attached to the PIM? Where no PIM has been requested it is to be attached to the building consent. Use condition 37 where applicable. Use warning note in GEMS. |  |
| Have the conditions/advice notes been added to the B-411? BCO to ensure conditions/advice notes have been correctly added to B-411. |  |
| Has an estimate been made of the inspections required on the B-103 to be added to the B-412 or B-413? |  |
| Where there are changes to specified systems has a draft compliance schedule been created on the B-462 for this building consent and the TRIM link sent to BuildingWOF@ccc.govt.nz? |  |
| Has all processing time and relevant fees been recorded on the B-103? |  |
| Has property hazard information received during processing been sent to EngineeringServicesGIS@ccc.govt.nz? Extract from file and save in TRIM as ABA******** Ground Report and send link to Engineering Services GIS. |  |
| Have actions been completed in GEMS and handling officer updated? |  |

BUILDING ACT, SECTION 50; Refusal of building consent Documented resolution of all processing sheets in this application is deemed not to satisfy on reasonable grounds that the provisions of the building code will be met if the building work is properly completed in accordance with the plans and specifications that accompanied the application. Reasons for refusing the consent have been forwarded to the applicant.

| Sign with CoSign Digital Signature when OK to refuse: |  |
| --- | --- |

BUILDING ACT, SECTION 49; Grant of building consent Documented resolution of all processing sheets in this application is deemed to satisfy on reasonable grounds that the provisions of the building code will be met if the building work is properly completed in accordance with the plans and specifications that accompanied the application.

| Sign with CoSign Digital Signature when OK to Issue: |  |
| --- | --- |

Enter autotext for processing sheets (“form number with no hyphen”). e.g. B110”

+

Form B-110

RESIDENTIAL 1 - 3

Residential 1 - Residential outbuildings and ancillary buildings – as defined by the Building Regulations 1992. Detached dwellings (SH) designed to a common standard (e.g. NZS 3604, NZS 4229) that are single storey and have an E2/AS1 risk matrix score less than or equal to 6.

Residential 2 - Detached dwellings (SH) designed to a common standard (e.g. NZS 3604, NZS 4229) that are less than or equal to two storeys and have an E2/AS1 risk matrix score less than or equal to 12.

Residential 3 - Detached dwellings (SH) or other dwellings (SR) that are less than or equal to three storeys but limited to vertical plane fire separation and direct egress to the outside. E2/AS1 risk matrix score of 13–20.

| Key for recording of decisions: |  |
| --- | --- |
| Y | Yes, meets all requirements, no comment required |
| OK | General intent satisfied, comments required |
| Q | Queried, fails to meet requirements, RFI sent |
| N/A | Not applicable but can also mean no to some questions |
| RES | Resolved after receiving further information |
| X (in heading) | For building code clauses not applicable to application |

| # | Items to be checked | Decision | Comments |
| --- | --- | --- | --- |
|  |  | Y, OK, Q, N/A, RES, X |  |
| GENERAL / PIM / SITE: |  |  |  |
|  | Has the Certificate of Title been checked for easements and consent notices that may affect the project? | na |  |
|  | Has the PIM / Development Report been checked for matters that may affect the building work? Consider protected trees and buildings, NZ Historic Places Trust, subdivision status, approvals needed, encumbrances, contamination, hazards, requirements for drainage, availability of services from NUO, health and dangerous goods implications. Use advice note 11 and /or 65 where subdivision is incomplete | ok | Dev report only, no issues eq repairs to ex foundations. |
|  | Has WebMap been checked for further matters, e.g. location of existing features, structures, hazards, services and outstanding consents that may affect the building work? The property file may need to be checked in some circumstances. | y | TC 2 |
|  | Has requests for utility connections on the application form and existing utility locations on WebMap or drainage plan been checked? | na |  |
|  | Do drainage outfalls require the connection to a common drain and have the common drain consent forms been completed? See Forms B-042 and B-043. | na |  |
|  | Has ground-bearing capacity and suitability of the site of the proposed building work been established? See Technical Decision 03/2009 for acceptance of CPEng PS1 without a ground report. Refer to Q29 for determination of ‘good ground’ for NZS 3604 foundations | ok | Soil report provided Sand soils 200kPa ult. @ 1.0m depths – recommends SED |
|  | Does the site plan show sufficient floor, ground and drain invert levels to establish compliance with B1, E1, E2, G13 and the City Plan? | y |  |
|  | Does site plan show boundary locations and dimensions, building setbacks to boundaries and other buildings? Consider also the need to show notional boundaries, easements and existing buildings. | y |  |
|  | Will a building location certificate be required? Hill site, close to planning limits or RC for RP or height. Use advice note 11, 27, 29 or 30 where applicable | na |  |
|  | Does the building work involve the removal or demolition of a building and has sufficient details been provided to be satisfied that this work will meet the provisions of the building code? Ensure it is included in the description of work. Use advice note 17, 19 and/or 22 where applicable | na |  |
|  | Has a sediment control management plan been provided that is relevant to project to achieve compliance with B1 and E1? See GD007. Use advice note 89 where applicable | y | 89 |
|  | Does the building work involve re-cladding where the original cladding has not met its durability? See GD003, advise Council’s Senior Building Support Officer. On buildings with residential use the building work may be part of WHRS remediation. | na |  |
|  | List all producer statements submitted with building consent application to support demonstrating compliance with building code. For all the producer statements not assessed by specialist review, record decision for acceptance. - Author / Engineer check - Building Code Clauses covered - Scope - Format / Type - Site Specific / Current - Inspections / Monitoring See Technical Procedure T-07. | ok | Sam Richards – CPEng B1 Foundation repair PS1 Site specific 1x excavations 1x pre pour 1x final level check Condition 2,4,49 Refers to attached report to site verification, eng to inspect anyway and provided PS4. Refers to report for plans covering – ok accept. |
| BUILDING ACT 2004: |  |  |  |
|  | Are there specific Building Act requirements to be considered? Place a Y in the table below next to any section that is applicable. Add related comments to the right-hand column Section 44 (2) Is this a staged consent? See Technical Procedure T-15 and add advice note C45 and where applicable C44.. Section 67-69: Is the building consent subject to a waiver or modification and have procedures for complying with the Building Act been followed? See GD019, T-23a. Use advice notes 70 or 71. Section 71-74: Is the land subject to natural hazards and have procedures for complying with the Building Act been followed? See GD013, T-12a, T-12b and T-12c. Use condition 55. Section 75-83: Is the building on two or more allotments and have procedures for complying with the Building Act been followed? See T-13, T-13a and T-13b. Use condition 53 or 54. y; Section 112: Is this an alteration to a building and have the procedures for complying with the Building Act been followed? Assess escape from fire and that the building will continue to comply with the provisions of the Building Code to at least the same extent as before the alteration. See T-14.1, GD034.. Section 113: Is the specified intended life nominated to be less than 50 years and has the procedures for complying with the Building Act been followed? See GD011, T-14.2. Use condition 46. Section 115: Is the proposed work a change of use and have procedures for complying with the Building Act been followed? Includes ‘incorporation in the building of 1 or more household units where household units did not exist before, the TA is satisfied, on reasonable grounds, that the building, in its new use, will comply, as nearly as is reasonably practicable, with the building code in all respects. See T-14.. Section 116: Is the proposed work an extension of life or subdivision of an existing building and have procedures for complying with the Building Act been followed? See T-14, T-14.3 and T-20.. Section 268: Does building consent application include any certified building methods or products? See DBH register http://dbh.govt.nz/product-certification-current-register. In Section 26, there is provision for the chief executive to issue a warning or ban regarding a building product or method. See T-43a.. Section 39 requires a TA to advise the NZ Historic Places Trust within 5 days after receiving an application for a PIM where that application affects a registered historic place See T-18. Usually considered in PIM review.. Section 122: Is this an earthquake-prone building and has the Territorial Authority policy been considered? Applies to residential buildings that are 2 or more storeys and have more than 3 household units. If yes, requires review by structural specialist.. | y |  |
| B1 - STRUCTURE: Note: Compliance document for Clause B1 – Structure, amendment 10 is still current until 31 January 2012 |  |  |  |
|  | Has the proposed building work had a specialist structural review on form B-150 and is it clear what parts were covered? | na |  |
|  | Are there parts of the proposed building work that are SED and are not being covered by specialist structural review? Identify items and whether adequate information on their construction is provided and if their designs covered under proprietary system tables or specific design PS-1. Use advice notes 2, 4, 49, 24 and/or 10 where applicable Roof trusses. Proprietary foundation systems. Proprietary buildings, e.g. kitset garages.. y; Specific foundation design to allow for less than good ground. Masonry walls. Structural framing members outside NZS 3604, e.g. beams, posts, etc. Steel framing - NASH standard see para 5.3, B1/VM1. Masonry veneer cladding, e.g. shelf angles, height, stack-bond. Other. | ok | Re levelling foundations by underpinning. |
|  | Are there ancillary structures in the proposed building work that are SED and are not being covered by specialist structural review? Retaining walls and gantries. Barriers and handrails fixing. Swimming pools. Signs, fences and hoardings. Decks and bridges. Other. | na |  |
|  | Have adequate construction details been provided of SED components to be satisfied the provisions of the Building Code will be met? | ok | Underpinning foundations To depths 1.0m as per soil report 200kPa bearing. H4 post – timber treatment ?? see below. 500x500 pad reinforced, provides stability Cast in 20mPa concrete. Jack and pack existing piles Timber packing H5 accept DPC between concrete and packer – accept Re fix connections concrete piles to timber bearers – Nailon plates with epoxied M10 anchors or 4mm wire stapled to timber bearers– accept 4mm wire 3604 fixing satisfied builder will be competent enough to will fix/install as required. |
|  | Are there parts of the building work that are designed using other Standards or Approved Documents from B1/AS1 and has adequate construction details been provided? Masonry NZS 4229. Earth Buildings NZS 4299. Stucco NZS 4251. Drains AS/ NZS 2566.1, AS/NZS2566.2, AS/NZS 2032 , AS/NZS2033. Glazing NZS 4223. Small Chimneys B1/AS3. Note, Timber barriers B1/AS2 removed from Amendment 11 | na |  |
|  | For additions and alterations to an existing building, have adequate details been provided of connection of structural elements to existing? Consider also that the provisions of section 112 or 115 are met for the existing building. | ok | Jack and packing ex piles as above – accept. |
|  | For building work designed to NZS 3604 have the correct design parameters been applied? c; Corrosion: Zone C / Zone D. m; Wind: Low / Med / High / VH / EH/ SED. 2; Earthquake: Zone 2 (all of CCC area within this zone). 1; Snow load: 1.0 kPa / 1.5 kPa/2.0 kPa/Specific Design. | y |  |
| NZS 3604 CONSTRUCTION: |  |  |  |
|  | Is the building within the scope and limitations of NZS 3604 as described in section 1.1.2? Provide details if NZS 3604 is used for part of a building | y | Ex build |
|  | Does the building site meet the requirements of 3.1.2 and has good ground been determined as per 3.1.3? Good ground, slope requirements and fill. Note the modification to the definition of good ground made by paragraph 3.1.1, B1/AS1 Note, that if a site does not comply with the definition of ‘good ground’ the foundations shall be SED | ok | Good ground not determined, sed as above. |
|  | Have other site requirements been considered? Bearing 3.4, site preparation 3.5, water in subfloor spaces 3.6, effects of tree roots on foundations 3.7 | ok | As above. |
| Durability: |  |  |  |
|  | Do timber and wood based products comply with section 4.3? Separation of timber from concrete/masonry, timber to NZS 3602, particle board flooring clearances. See also TD 04/2009. | ok | DPC between – accept. H4 post to underpinning foundation?? Facilitate consent, other notes on plan state H5 use. Add note. H5 treated timber when in contact with the ground to meet NZS3602. H5 timber packing to piles - ok |
|  | Do steel fixings and fastenings comply with section 4.4? See TD 21/2008. | ok | Pile/bearer fixings spec’d, concrete piles. |
|  | Does concrete and concrete masonry comply with section 4.5? | y | 20mPa |
| Bracing Design: |  | NA |  |
|  | Have the correct parameters been used for determining bracing demand? Wind 5.2, Earthquake 5.3 or Gib EzyBrace Systems. |  |  |
|  | Where applicable does sub-floor bracing design comply with section 5.5? Distribution, bracing element types, calculations. | NA | Existing piles/bearers to remain pack only. Small area indicated. braced pile connections re-instatement. 6kN in each direction required. Add note to cover, sat builder will install comply, inspector can check. |
|  | Does wall-bracing design comply with section 5.4? Distribution, bracing element types, calculations. |  |  |
|  | Where diaphragms have been used with a brace wall system do they comply with section 5.6? |  |  |
|  | Where wall-bracing design involves reinforced concrete or concrete masonry walls and/or dragon ties do they comply with section 8.3? |  |  |
| Foundation & Sub-Floor: |  | NA |  |
|  | Does the sub-floor system and the set out of the sub-floor system comply with sections 6.2 & 6.3? Consider load paths through the building and spans of bearers and floor joists. |  |  |
|  | Do the foundation piles comply with section 6.4? Height, materials, footings. |  |  |
|  | Do the different types of piles comply with the relevant sections 6.5 – 6.9? Ordinary 6.5, driven timber 6.6, cantilevered 6.7, braced 6.8, anchor 6.9? |  |  |
|  | Do framed sub-floor walls comply with section 6.10? |  |  |
|  | Do concrete or concrete masonry walls comply with 6.11? Height, width, footing, lateral support, reinforcing, fixing of wall plates. |  |  |
|  | Do bearers comply with section 6.12? Fixing to foundation walls, sizes, cantilevers, joints. |  |  |
|  | Do stringers comply with section 6.13? Dimensions, fixing. |  |  |
|  | Does the prevention of dampness measures comply with of section 6.14? Openings, ground cover, access, horizontal separation. |  |  |
| Suspended Timber Floors: |  | NA |  |
|  | Do floor joists comply with section 7.1? Sizes, joints, lateral support, supporting walls, connected to sub-floor bracing, cantilevered, trimming, holes and notches. See section 14 for 3 kPa floor loads. |  |  |
|  | Does the flooring comply with section 7.2? |  |  |
|  | Do flooring diaphragms comply with section 7.3? |  |  |
|  | Do timber decks comply with section 7.4? Design, bracing, decking. |  |  |
|  | Where the design of the building includes 3kPa floor loads has the increased requirements of section 14 been met? Includes bracing demand, subfloor members, floor joists, lintels, wall framing |  |  |
| Concrete Slab-on-ground Floors: |  | NA |  |
|  | Do the size limitations, ground clearances and foundation edge details of concrete floor slabs comply with sections 7.5.1 and 7.5.2? |  |  |
|  | Do the granular base and DPM to concrete floor slabs comply with sections 7.5.3 – 7.5.7? |  |  |
|  | Do the thickness, shrinkage control, bearing, support of internal walls and fixing of timber to concrete floor slabs comply with sections 7.5.8 – 7.5.12? |  |  |
| Walls: |  | NA |  |
|  | Does the wall framing meet the general requirements of section 8.4 and studs in wall framing comply with section 8.5? Inclined not more than 20º from vertical. Sizes, spacings, holes and notches, trimming. |  |  |
|  | Do lintels, sill and head trimmers comply with section 8.6? Sizes, sill and head trimmers, fixing. |  |  |
|  | Do top and bottom plates, dwangs and ribbon boards comply with sections 8.7 and 8.8? Sizes, joints, holes and notches, lateral support including gable ends |  |  |
| Posts: |  | NA |  |
|  | Do posts comply with section 9? Length, uplift, connections. |  |  |
| Roof Framing: |  | NA |  |
|  | Does the roof framing design comply with section 10.1? Excludes flat roofs with access and roofs under minimum pitch required by NZBC E2/AS1. |  |  |
|  | Do framed roofs comply with section 10.2.1 Rafters – 10.2.1.3, Ridge boards – 10.2.1.4, Ridge beams – 10.2.1.5, Ceiling Joists – 10.2.1.6, Ceiling runners – 10.2.1.7, Valley boards – 10.2.1.8, Underpurlins 10.2.1.9, Underpurlin struts – 10.2.1.10, Strutting beams – 10.2.1.11, Verandah beams – 10.2.1.12, Collar ties & cleats – 10.2.1.13, Dummy rafters – 10.2.1.17 |  |  |
|  | Do eaves and gable verges comply with sections 10.2.1.14 and 10.2.1.15? |  |  |
|  | Do purlins and tile battens comply with section 10.2.1.16? |  |  |
|  | Where roof construction is specific design trusses does it comply with section 10.2.2? Maximum dimensions and spacings – 10.2.2.2, Drawings and specifications – 10.2.2.3, Fixings – 10.2.2.6, Roof truss bracing – 10.2.2.7. Point loads from girder trusses require specific design. Advice note 10 to be used where an ‘as built’ is required. |  |  |
|  | Do the roof bracing systems and roof bracing details comply with section 10.3 and 10.4? Different requirements for light hip, heavy hip, light gable or heavy gable. |  |  |
| Ceilings: |  | NA |  |
|  | Do ceiling lining supports comply with section 13.2? 13.2.1 Truss roofs, 13.2.2 framed roofs and floors. |  |  |
|  | Do openings in ceilings and support of water tanks in roof spaces comply with section 13.3 and 13.4? |  |  |
|  | Do structural ceiling diaphragms comply with section 13.5? |  |  |
| Snow Loading: |  | NA |  |
|  | Have all structural members subject to snow loading of up to 1.5 kPa or up to 2.0 kPa been sized and factored to tables in section 15? 1.0kPa up to 100 metres altitude use general tables in sections 8 and 10, 1.5kPa up to 200 metres altitude, 2.0kPa up to 350 metres altitude, SED over 350 metres altitude |  |  |
| B2 - DURABILITY: |  |  |  |
|  | Does material compatibility comply with Tables 21 and 22, E2/AS1? | Y |  |
|  | Do woodbased building components required to achieve a 50 year durability comply with Table 1 NZS 3602:2003 as modified by B2/AS1? Timber treatment of structural elements. | OK | Q above. |
|  | Do woodbased building components required to achieve a 15 year durability comply with Table 2 NZS 3602:2003 as modified by B2/AS1? Timber treatment of claddings, etc. | na |  |
|  | Has an adequate coating system been specified for exposed structural steel work? | na |  |
|  | Can maintenance be carried out as necessary to achieve the required durability of the cladding system? See 2.5, E2/AS1. | na |  |
| C1 – OUTBREAK OF FIRE: |  | NA |  |
|  | Where the installation of a solid fuel heater is proposed have manufacturer’s installation specifications been provided that have been verified using AS/NZS 2918 to comply with C1.3.2? Or check that it has been pre-approved by CCC TD 06/2008. |  |  |
|  | Is the solid fuel burner approved for use on this property under ECan’s Air Plan? Check approved burners list and rules on ECan’s website. Ensure the solid fuel burner details are correctly recorded under ‘description of work’ in GEMS. |  |  |
| C2 – MEANS OF ESCAPE: |  |  |  |
|  | Does the escape route length from any part of the house have a maximum dead end length of 24m? See 3.5, C/AS1. Consideration of escape routes to safe places in SR purpose group buildings See 3.14, C/AS1 | y | No change |
| C3 – SPREAD OF FIRE: C4 – STRUCTURAL STABILITY DURING FIRE: |  | NA NA |  |
|  | Where a building in a household unit does not meet the required fire separation from a title, cross lease or notional boundary or another household unit has a correctly detailed 30/30/30 two-way FRR wall been provided? See B-312 SH information sheet. FRR required for wall up to 1m of boundary or if the eave (includes the gutter and spouting) is within 650mm. Including fire wall support, fire rating is to extend to underside of eaves and non-combustible cladding. |  |  |
| D1 – ACCESS ROUTES: |  | NA |  |
|  | Do access routes have adequate activity space, are free from dangerous obstructions, have a safe cross fall, a safe slope in the direction of travel and adequate slip resistant walking surfaces? See Section 1,2 & 3, D1/AS1. For housing, only stairs and the route to the main entrance need to be considered. |  |  |
|  | Do access routes include stairs to upper floors, have stair treads that provide adequate footing, have an uniform rise, treads that can be easily seen and prevent children becoming trapped and have appropriate landings? See Section 4 & 5, D1/AS1. |  |  |
|  | Do access routes have adequate handrails to assist movement on stairs and ladders? See Section 6, D1/AS1. |  |  |
| D2 – MECHANICAL INSTALLATION FOR ACCESS: |  | NA |  |
|  | Where applicable has this been covered by specialist review on form B-160? |  |  |
| E1 – SURFACE WATER: |  | NA |  |
|  | Where compliance is being shown with an alternative solution or SED to E1/VM1 has this been covered by a civil specialist review? Use advice note 6 where applicable |  |  |
|  | Are floor levels above the minimum required by the PIM/Development Report or Section 2, E1/AS1to establish compliance with E1.3.2? Note: Limits on application to E1.3.2 Flood-prone sites may need SED and/or natural hazard notice. See TD 09/2009.. |  |  |
|  | Is there an approved stormwater outfall identified? Check PIM for requirements and drainage records for any existing outfall. ECan approval may be required in some cases, e.g. Banks Peninsula. See information sheet B-330 Where soakage is proposed as an outfall is has to be an approved area and have ground suitability assessed with a disposal test. Use advice note 1 where applicable |  |  |
|  | Has a layout of drainage to convey surface water to an appropriate outfall been provided, with sufficient details of pipe sizes, gradients, access points, invert levels and capacity of existing system, to establish compliance with E1.3.3? See Table 2, Fig. 3, 10 - 12, Section 3.2 & 3.7, E1/AS1. For bubble-up chambers see Fig. 6 & 7, section 3.4.2, E1/AS1 Consider access to drains and flushing points on retaining walls. |  |  |
|  | Has the implications of trenches close to buildings and drains under buildings been considered? See Section 3.7.6 and 3.9.7, Fig 14, E1/AS1. |  |  |
|  | Have adequate details of surface water control for paved areas been provided? See TD10/1996, TD01/1999 res only and see Fig 8 and 9, Section 3.6.2, E1/AS1. Type 1 up to 90m2 Type 2 up to 800m2. See TD11/2008. Use advice note 85 where applicable |  |  |
|  | Have details of sufficient down pipes at an adequate size been provided? See table 5, E1/AS1. |  |  |
|  | Have the internal and external gutters been adequately sized and is there adequate provision for overflow from internal gutters? See Fig 15 and 16, Section 5.1, E1/AS1. and Section 5.5.1, E1/AS1 |  |  |
| E2 – EXTERNAL MOISTURE: Note: Compliance document for Clause E2 – amendment 4 is still current until 31 January 2012 |  | NA |  |
|  | Is the building design within the scope of the acceptable solutions to E2? E2/AS1. E2/AS2 – Earth buildings. E2/AS3 - Concrete and concrete masonry buildings. See section 1.1, E2/AS1. See CCANZ CP 01:2011 for E2/AS3 |  |  |
|  | Is the roof cladding system within the limitations of E2/AS1 and does it comply with the relevant section: Masonry tiles (section 8.2) Pitch, profiles, anti-ponding, figures 23 – 31, table 10. Pressed metal tiles (section 8.3) Pitch, profiles, figures 32 – 37.. Profiled metal roof claddings (section 8.4) Pitch, profiles, length, figures 38 – 55, tables 11 – 17.. Membrane roofing (section 8.5) Falls, substrate, junctions with walls, outlets, figures 56 – 64.. |  |  |
|  | Is the wall cladding system within the limitations of E2/AS1 and does it comply with the relevant section: Masonry veneer (section 9.2) Figure 73, see also SNZ HB 4236, shelf angles are SED. Also use advice note 52 for shelf angles. Stucco (section 9.3) Figures 74 – 76, see also NZS 4251. Use advice note 51. Timber weatherboard (section 9.4) Profiles –horizontal bevel-back, rebated bevel-back, and rusticated, vertical shiplap and board and batten. Figures 77 – 86.. Fibre Cement Weatherboard (section 9.5) Laps and joints, figures 87 – 91.. Profiled Metal Wall Cladding (section 9.6) Horizontal profiled over drained cavity, vertical profiled direct fixed over roof underlay, Fig 92 - 102. Fibre Cement Sheet (section 9.7) Flush-finished joint systems or other joint systems see 9.7.1, figures 103 – 117, Table 19.. Plywood Sheet (section 9.8) Figures 118 – 123. EIFS (section 9.9) Use advice note 50. |  |  |
|  | Are alternative solutions for compliance with E2 being used for roof or wall cladding systems? - Buildings, components or junction details outside the scope of this Acceptable Solution require specific design. Details of specific design shall be provided to the building consent authority for assessment and approval as part of the building consent process. Section 1.4, E2/AS1. - Check for recognized appraisals for cladding systems. Check proposed details of these cladding systems against appraised product literature. - DBH publication - External Moisture - An introduction to weathertightness design principles for guidance to more complex assessments. - For metal roof cladding see also TD 05/2008 - NZ Metal Roofing and Wall Cladding Code of Practice. Proprietary torch on membranes. Proprietary lightweight cladding. Proprietary AAC panel system. See also TD 14/2009 and TD 17/2009.. Other. |  |  |
|  | Have details not covered in E2/AS1 been assessed to comply with the performance requirements of E2? See BRANZ publications - Weathertight solutions series. Junctions between different types of claddings. Skillion roofs. Timber framed windows/doors. Basements (note removed from E2/AS1 amendment 5). Other. |  |  |
| E2 / AS1: |  |  |  |
|  | Is there provision for preventing the ingress of snow melt water? Consider snow boards to internal gutters >40m above sea level. |  |  |
|  | Has a risk matrix score using section 3 been provided for elevations using acceptable solution wall cladding systems? Figures 1 – 4, Tables 1 – 6. |  |  |
|  | Do flashings details comply with the general requirements of section 4? Materials, durability, laps, dimensions see table 7, figures 5 – 7. |  |  |
|  | Do roof/wall junction details comply with the general requirements of section 5? Apron flashings, gutters, barges, fascias and their junctions to walls, figures 8.A & 8B |  |  |
|  | Do parapet details comply with the general requirements of section 6? Materials, capping, slope, wall junctions, 6.6 integral surface cappings not for parapets, figures 9 – 13. |  |  |
|  | Do decks and pergolas attached to the building comply with the general requirements of section 7? Thresholds, attachment, level thresholds, enclosed balustrades, figures 14 – 19. |  |  |
|  | Does the roof underlay comply with the general requirements of section 8.1.5? Table 23, underlay support, see also GD035. |  |  |
|  | Do internal, valley and hidden gutters comply with the general requirements of section 8.1.6 E2/AS1? Materials, dimensions catchment, spreaders, figure 20, table 8. |  |  |
|  | Do roof penetration details comply with the general requirements of section 8.1.7, E2/AS1? Support, catchment, figures 21, 22, table 9. |  |  |
|  | Does the bottom of wall cladding comply with the general requirements of section 9.1.3? Table 18, figure 65. |  |  |
|  | Do barriers to airflow, wall openings, air seals and building wrap comply with the general requirements of sections 9.1.4 – 9.1.7? See table 23 and GD035. |  |  |
|  | Do drained cavities comply with the general requirements of section 9.1.8? Figures 66, 67. |  |  |
|  | Do wall penetrations comply with the general requirements of section 9.1.9? Figures 68 – 70. |  |  |
|  | Do windows and doors comply with the general requirements of section 9.1.10? Figures 71, 72A & 72 B. |  |  |
|  | Has the potential for unacceptable construction moisture been assessed with section 10? |  |  |
| E3 – INTERNAL MOISTURE: |  | NA |  |
|  | Have details of an adequate combination of thermal resistance, ventilation, and space temperature been provided? See E3.3.1 and Section 1 E3/AS1. Note: Does not apply to communal non-residential, commercial, industrial, outbuildings, or ancillary buildings. Where steel structure has been used in the building envelope ensure an adequate thermal break will be provided |  |  |
|  | Has provision been made where accidental overflow could damage an adjoining household unit? See E3.3.2 and Section 2, E3/AS1. NB Does not apply where the separating wall is concrete or concrete masonry. |  |  |
|  | Have details been provided for surfaces to be impervious, easily cleaned and will prevent water penetration into concealed spaces where adjacent to sanitary fixtures and appliances or likely to be splashed? See E3.3.3, E3.3.4, E3.3.5, E3.3.6 and Section 3, E3/AS1. |  |  |
|  | Where level access showers and/or tiled shower enclosures are proposed, have sufficient details of gradients to floor, substrates, surface finishes and tanking membrane been provided? Consider durability of substrate in wet areas where maintenance of impervious cannot be assured See TD 04/2009 and 110.3.1, NZS3602. |  |  |
| F1 – HAZARDOUS AGENTS ON SITE: |  | NA |  |
|  | Where contamination of site has been identified has this been covered by contamination specialist review on form B-154? See GD009, TD 02/2008. |  |  |
| F2 – HAZARDOUS BUILDING MATERIALS: |  | NA |  |
|  | Has sufficient details been provided to ensure Grade A safety glazing will be used where required by NZS 4223 Part 3 as modified by F2/AS1? Generally windows supplied by window manufacturers will have glazing selected by design software. Consider existing windows where the risk has increased. |  |  |
|  | Has an investigation been done and/or a procedure plan been submitted for existing buildings where it is reasonably likely asbestos in a potentially hazardous form will be disturbed by the proposed building work? |  |  |
| F3 – HAZARDOUS SUBSTANCES AND PROCESSES: |  | NA |  |
|  | Liquid fuel burners, consider bunding and location of storage tanks. Installation to AS1691. Any consent with over 100kg gas or 1500L diesel requires specialist review on form B-156. |  |  |
| F4 – SAFETY FROM FALLING: |  | NA |  |
|  | Has a barrier been provided where people could fall 1 metre or more from a floor of a building, from an opening in the external envelope or from a sudden change of level within or associated with a building including roofs with permanent access? See F4.3.1 & F4.3.2 and limits on application for F4.3.1, Section 1, F4/AS1 & Section 2, F4/AS1.. See also GD004. |  |  |
|  | Do the barriers meet all the requirements of F4.3.4? See F4/AS1 for acceptable barrier height and construction. TD 02/2006, TD 24/2008 and TD 11/2009. |  |  |
|  | Where applicable have swimming pool fences been reviewed on B-152(a)(b)? Also requires review from Council Swimming Pool Inspector. 1.2.7 The Schedule to the Fencing of Swimming Pools Act 1987 is a means of establishing compliance with NZBC Clause F4. Use advice note 74 |  |  |
| F5 – CONSTRUCTION & DEMOLITION HAZARDS: |  | NA |  |
|  | Have details been provided of barriers for the protecting the public and for restricting access by children to the site that are relevant to the potential work-site hazards? Note: Barriers are not necessary for domestic dwellings up to 2 storeys above ground level unless specific hazards exist. |  |  |
| F7 – WARNING SYSTEMS: |  |  |  |
|  | Has smoke alarm details been provided to comply with section 3, F7/AS1? In, or within 3.0m of, bedrooms and on escape routes from building. TD 10/2008. See also information sheet B-311. | ok | Not on plans, satisfied will comply, inspector can check on site. |
| G1 – PERSONAL HYGIENE: |  | NA |  |
|  | Does the layout show adequate space dimensions and does the location of spaces containing soil fixtures show adequate separation from other spaces and have adequate provision for hand washing? See G1.3.2 and Section 3, G1/AS1. |  |  |
| G2 – LAUNDERING: |  | NA |  |
|  | Have complying laundry facilities and service connections been provided? Section 1, G2/AS1. |  |  |
| G3 – FOOD PREPARATION & PREVENTION OF CONTAMINATION: |  | NA |  |
|  | Does the layout show provision of a sink with waste, hot and cold water supply, pantry or fridge, cooking appliance, hygienic food preparation area and do wall linings adjacent to appliances and facilities have surfaces that can be easily maintained in hygienic condition? See G3.3.1, Section 1, G3/AS1 |  |  |
| G4 – VENTILATION: |  | NA |  |
|  | Do spaces within buildings have means of ventilation with outdoor air that will provide an adequate number of air changes to maintain air purity? See G4.3.1 and Section 1, G4/AS1. |  |  |
|  | Do buildings have a means of collecting or otherwise removing moisture and other contaminants from the spaces in which they are generated? See G4.3.3 and TD 01/1997. |  |  |
|  | Has consideration been given to providing extra ventilation for gas-fuelled appliances? See G4.3.5 and Section 2, G4/AS1. NB for dwellings it is unlikely that compliance will not be met. |  |  |
| G6 – AIRBORNE & IMPACT SOUND: |  | NA |  |
|  | Is the STC rating of common walls, floors and ceilings of habitable spaces in household units between occupancies no less than 55? See G6.2, G6.3.1, G6/AS1. Prevent noise transmission only to the habitable spaces of household units. Note: Where alternative solution is proposed or sound insulation is required under City Plan then use specialist review on form B-155. |  |  |
|  | Is the IIC rating of the floors to a separate occupancy above the habitable space of a household unit no less than 55? See G6.2, G6.3.2 G6/AS1 Prevent noise transmission only to the habitable spaces of household units. |  |  |
| G7 – NATURAL LIGHT: |  | NA |  |
|  | - Are habitable spaces provided with adequate openings for natural light? See G7.3.1, section 1.0 G7/AS1. |  |  |
|  | Are habitable spaces provided with adequate openings for visual awareness of the outside environment? - See G7.3.2, section 2.0, G7/AS1. |  |  |
| G8 – ARTIFICIAL LIGHT: |  | NA |  |
|  | Are spaces used by people, provided with adequate artificial light to enable safe movement in the absence of natural light? Check that lighting is to be provided to exitways, access routes and common areas. |  |  |
| G9 – ELECTRICITY: |  | NA |  |
|  | Has sufficient information been provided that the building work will meet the functional requirement ‘where provided in a building, electrical installations shall be safe for their intended use’? Use advice note 99 NZECP 51 is an Acceptable Solution for electrical installations within domestic dwellings. TD 09/2008 (or TD 06/2008?) Use advice note C99. |  |  |
| G10 – PIPED SERVICES G11 – GAS AS AN ENERGY SOURCE: |  | NA NA |  |
|  | Is there gas appliances being installed and does documentation nominate compliance with G10/AS1, G11/AS1 and NZS 5261? See TD 09/2008 (or TD 06/2008?) Use advice note C99 or C100 where applicable. |  |  |
|  | Does the LPG cylinder location comply with App G, NZS 5261? Any consents with over 100kg gas requires specialist review. |  |  |
| G12 – WATER SUPPLIES: |  | NA |  |
|  | Is the water supply from a potable source? G12.3.1. See information sheet B-313. Well supply, rainwater, or streams require a potable water test. Note also, ECan approval may be needed to extract water. Use advice note 76 where applicable |  |  |
|  | Is the potable water supply isolated from non-potable and are non-potable water supply outlets and pipelines identified? G12.3.2, G12.3.4, sections 3 & 4 G12/AS1. |  |  |
|  | Is the potable water supply being protected from cross connection or backflow with air gap or backflow prevention device? G12.3.2, section 3,G12/AS1, see GD027. |  |  |
|  | Does the hot & cold water supply system comply with section 5 & 6, G12/AS1? G12.3.5, G12.3.6, G12.3.7, G12.3.8, G12.3.9 Includes water tanks, pipe sizes, isolating valves, water temperature at hygiene fixture and prevention of the growth of legionella bacteria.. |  |  |
|  | Is the storage water heater adequately supported and restrained, and located where it can be accessed for inspection, maintenance and removal? Fig 14 and Section 6.11, G12/AS1. HWC in roof space may require SED. Also consider maximum pipe length from HWC to kitchen sink outlet, see H1.3.4, Section 5, H1/AS1 & NZS 4305 |  |  |
|  | Where a solar water heater is being installed has it been reviewed on form B-112? Use advice note S1 where applicable |  |  |
| G13 – FOUL WATER: |  | NA |  |
|  | Has a layout of sanitary plumbing to convey foul water to a drainage system been provided with sufficient details of pipe sizes, gradients and venting to establish compliance with G13.3.1? See Sections 3 - 5, G13/AS1 or Sections 6 – 8 AS/NZS 3500.2.. Consider also floor wastes outlets and routing of pipe work through the structure of the building to comply with B1. |  |  |
|  | Has a layout of drainage to convey foul water to an appropriate outfall been provided with sufficient details of pipe sizes, gradients, access points, ventilation, invert levels, overflow relief and capacity of existing system to establish compliance with G13.3.2? See Sections 3 - 5, G13/AS2 or Sections 3 & 4, AS/NZS 3500.2. Consider overflow relief, see 3.3.2, G13/AS2 or 4.6.6.1, AS/NZS 3500.2 as modified by 1.0.3, G13/AS3. See also GD024. Use advice note 88 where applicable |  |  |
|  | Has the implications of trenches close to buildings, unstable soils and drains under buildings been considered? See sections 5.2, 5.3, 5,6, 5.8 & 5.9, G13/AS2. |  |  |
|  | Where a pumped discharge is proposed have sufficient details been provided to establish compliance with G13.3.2 and G13.3.3? See section 10, AS/NZS 3500.2 and NUO requirements. |  |  |
|  | Where an on-site disposal system is being installed has it been adequately designed to comply with the performance clause G13.3.4? G13/VM4 references a design method; given in Part 4 of AS/NZS 1547, Generally a proprietary system will be specified. Note: ECan approval may be needed for an on-site disposal system. |  |  |
| H1 – ENERGY EFFICIENCY: |  | NA |  |
|  | Is the building’s thermal envelope to its conditioned space defined and airflow controlled? See H1.2 (a) and H1.3.1, section 3.0, H1/AS1. Consider garages and conservatories. |  |  |
|  | Has the method of compliance for adequate thermal resistance of the building’s thermal envelope been nominated and shown to comply? See H1.3.1, H1.3.2 and H1.3.3, H1/VM1, H1/AS1 and GD033. Schedule method, NZS 4218:2004 as modified by H1/AS1. Calculation method, NZS 4218:2004 as modified by H1/AS1. Modelling method, NZS 4218:2004 as modified by H1/VM1. BPI verification method, e.g. ALF3.2.. Alternative solution ( e.g. Appendix D, NZS 4218:2009 for additions). |  |  |
|  | Will the construction and insulation specified on the consent drawings achieve the construction R-value stated in the compliance report? Check with BRANZ House Insulation Guide or NZS 4214. Consider depth of the construction cavity, 25mm air gap from roof underlay, exposed sub-floors, and thermal bridging. Also extra requirements for embedded heating systems see Replacement Table 3 - H1/AS1 |  |  |
| OTHER ITEMS: |  |  |  |
|  | Has an adequate specification that is relevant to the project been provided covering all trades, materials, proprietary systems and reference documents? Note, the specifications should not contain irrelevant information or contradict the drawings. | ok | On plans small scope |
|  | Record any decisions regarding other issues e.g. - Durability to satisfy functional requirements of the Building Code for all clauses throughout the life of the building. ## Existing building to continue to comply with the provisions of the Building Code to at least the same extent as before the alteration. ## Compliance schedule - a building used wholly as a single household unit requires a compliance schedule only if it has a cable car attached to it or servicing it. Note: A draft compliance schedule (B-462 section 1) is required to be issued with the building consent. | ok | ## Existing building to continue to comply with the provisions of the Building Code to at least the same extent as before the alteration. |