Christchurch City Council

Environmental Policy & Approvals Unit

Engineering Conditions

2 November 2006

From:	Nigel Baker (Subdivisions Engineering Officer)

To:	Angela O'Reilly (Subdivision Assistant)

- SUBDIVISION CONSENT APPLICATION (FEE SIMPLE)

- SUBDIVISION REF		RMA 92006653

- APPLICANT			Jesson & Mercer Ltd

- CONSULTANT		balfour Taylor

- Consultant REF	2915

- ADDRESS OF ACTIVITY	97 Sullivan Ave

Sullivan Ave 97 – Jesson & Mercer s/d – RMA 92006653 – Application Report & Plan

## ENGINEERING CONDITIONS

### General

3.1	The minor drainage works proposed for this subdivision consent must be installed and inspected under a building consent obtained from the Environmental Policy and Approvals Unit.

Form BA2M for work under $20,000: http://www.ccc.govt.nz/building/forms/ba2m.pdf

Form BA2 for works over $20,000  http://www.ccc.govt.nz/building/forms/BA2.pdf

Service Connections (sewer & stormwater) to Council Services in the street are authorised work and must be carried out by a Council authorised drainlayer. This includes all drainage laterals on roads, footpaths and verges that connect the property to public drains.

A list of Council authorised drainlayers is available on request or online at website www.ccc.govt.nz/WasteWater/AuthorisedDrainLayers/.

For further information the applicant is advised to contact either Tony Borkus (941-8376) or Gordon Taylor (941-8375.)

### Water Supply

4.1	Any existing reticulation between lots 1 and 2 must be removed.

### Sewage

- 5.2	A sanitary sewer lateral is to be laid to at least 600mm inside the net site area of Lot 2 at the subdivision stage.

5.3	The existing sewer pipe currently serving Lot 1 may be extended to serve Lot 2 providing:

- Any extension laid must be to a depth sufficient to serve the furthermost part of the building site and to any potential new premises.

- The condition of the existing pipe is satisfactory.

### Stormwater

- 6.1	A stormwater lateral is to be laid to at least 600mm inside the building area of Lot 2 at the subdivision stage.

6.2	Stormwater from existing roof and hard-stand areas is to be piped to the kerb and channel at the road frontage unless currently discharged to an approved outfall.

### Access

8.1	The access to Lot 2 is to be constructed in accordance with Rule 14 5.2.2(b) and (c) of the Proposed District Plan to the standards embodied in Council’s Code of Urban Subdivision.

The minimum standard of formation required for the access is to be formed and metalled.

The formation required for ‘Formed and Metalled’ is as follows:

- A minimum formed width of 2.7m.

- A minimum depth of 150mm of compacted metalcourse.

- 100x40 H4 Treated Timber Battens on both sides of the formation.

- The formation is to be adequately drained.

8.2	Photographs are to be supplied that clearly show a view or views (including the vehicle crossing) of the formation that the Subdivision Team can identify as complying with the access construction condition of the consent.

8.3	The consent holder shall construct access for rear lots from the road carriageway to the road frontage in accordance with the Council’s Construction Standard Specification Part 6, Clause 6 and Standard Details SD606, SD607, SD608, SD611, SD612, SD615 & SD616. For new formation, Clegg hammer test results complying with clause 6.5 ‘Metalcourse’ are to be supplied with the 224c Conditions Certificate request.

## Development Contributions (2006)

Development Contributions under the LTCCP (2006) for Network Infrastructure are :

Water supply and conservation		DWS	$596.79

Wastewater collection			DWC	$505.49

Wastewater treatment and disposal	DWT	$645.40

$1,747.68 (Total including GST)

For one new lot.

Note that the development contributions ay be paid in conjunction with building consent 10070974. If these charges have already been paid then evidence of the payment of these amounts is required prior to the issue of the section 224 conditions certificate for the subdivision consent.

## INFORMATION FOR THE CONSENT HOLDER

### Water Supply

A new water supply connection will be required for lot 2 and this will need to be applied for as part of the building consent. Note that the new meter is to be connected to the 100mm main as 25mm sub-main has insufficient capacity for another meter.

### Sewerage

The site is considered satisfactory at existing ground levels.

### Minimum Levels and Filling

Minimum site levels have been set at building consent stage.

The consent holder is required to mitigate any adverse affects on adjoining properties. Failure to undertake such works may mean that the consent holder has some responsibility to adjacent owners at law.

Nigel Baker

Subdivision Engineering officer

Ext 8318