Application for Resource Consent
Joint decision report in accordance with Sections 95-95e and 42a of the Resource Management Act 1991

| Resource consent reference number: | LL-2020-9257-00 |
| --- | --- |
| Applicant: | RAYNES, BARRY |
| Agent: | FERGUSSON PLANNING LIMITED |
| Proposal: | Dwelling with non-compliant outdoor living, site permeability, and building platform of garage |
| Site address: | 60A POHUTU STREET, WHAKATĀNE |
| Legal description of site: | Lot 2 DP 468623 |
| Site area: | 469 square metres |
| Zoning: | Residential |
| Special features / policy areas / hazards or references on the RT: | 11500541.1 – Section 72 11847707.1 - Consent Notice Flooding (1% AEP flood level 102.0m Whakatane Datum) |
| Site visit: | 21 October 2021 |
| Notification recommendation: | Non-notified |
| Recommendation: | Grant with conditions |
| Reporting planner: | Rebecca Hart |

# proposal

  - Land use consent is sought for minor non-compliances relating to the construction of a proposed dwelling. The dwelling is to be constructed on a vacant rear residential site at 60A Pohutu Street. The site is a flat residential allotment and was created through an infill subdivision (SS-2019-9001-00).

  - A consent notice (figure 6) has been imposed on the title of the property requiring stormwater and geotechnical assessments be provided as part of the building consent for any dwelling on the property. The property is subject to inundation hazards and has a Section 72 notice on the title (figure 3).

  - The proposal requires resource consent due to non-compliances with the following performance standards listed in the Whakatāne District Plan:

- Rule 4.2.7 – Outdoor Living Space

- Rule 4.2.10.3 – Site Permeability

- Rule 18.2.3.2 – Flooding (building platform level of garage)

  - The description below (figure 1) has been extracted from resource consent application submitted by Tim Fergusson, Fergusson Planning, November 2020, and the revised site plan below (Drawing 101) was submitted along with the section 92 response (figure 2) on 20 October 2021. Note that the extent of the non-compliance with site permeability is actually 65.6% as noted on the site plan.

  - Timeframes

| - Date lodged | - 20 November 2020 |
| --- | --- |
| - Further information requested (s92) | - 2 December 2020 |
| - Further information received (s92) | - 20 October 2021 |
| - Draft conditions of consent sent to applicant (s37A) | - 26 October 2021 |
| - Applicant comments on draft conditions |  |
| - Combined report forwarded for peer review | - 22 October 2021 |
| - Other… e.g. s88, s95e |  |

Figure 1: Application Proposal (source; Resource consent application submitted by Fergusson Planning, November 2020)

Figure 2: Revised site plan received 20 October 2021 (Drawing 101, dated 27/09/2021) showing location of detention tanks on the western boundary.

Figure 3: Excerpt from the Record of Title

# Description of site and surrounds

  - The following description has been extracted from resource consent application submitted by Fergusson Planning, November 2020:

Figure 4: Aerial image of 60a Pohutu Street. Source: Whakatāne District Council GIS maps

Figure 5: Photos of vacant subject site – 60A Pohutu Street.

Figure 6: Consent Notice - 11847707.1

# Relevant rules/activity status

  - The application must be assessed under the Operative Whakatāne District Plan (the District Plan). The property is zoned RESIDENTIAL in the District Plan.

  - An assessment of the proposal’s compliance with the relevant rules of the District Plan has been completed. The table below outlines the relevant rules.

Table 1: Summary of relevant rules

| - Rule # | - Rule name | - Status of activity | - Comment |
| --- | --- | --- | --- |
| - 3.4.1.1 – 1. | - One dwelling per lot | - Permitted | - The proposal is seeking resource consent to construct one dwelling on a vacant lot which does not meet all of the relevant performance standards as set out below. |

  - The proposal complies with the following performance standards for the Residential Zone: height, natural light, distance to boundary, density of residential dwellings, outdoor service courts and visual privacy.

  - The resource consent application notes compliance with the following relevant performance standards: earthworks (11.2.2), noise (11.2.6), onsite parking (13.2.9), wastewater (13.2.27.1, stormwater (13.2.28), water (13.2.29).

  - The amended plans on Drawing 106 (received on 2 August 2021) show that the deck height has been reduced to 950mm, so that it does not fall under the definition of a ‘building’, and therefore complies with boundary setbacks.

  - The table below provides a summary of the rules that are not met and trigger the requirement for a resource consent.

Table 2: Summary of relevant zone standards

| - Rule # | - Rule name | - Status of activity | - Comment |
| --- | --- | --- | --- |
| 4.2.7 - Outdoor living space | Each dwelling shall have outdoor living space which shall; if located at ground level (see Figure 4.6, Figure 4.7 and Figure 4.9); i. have a minimum area of 40m² in the Residential Zone and a minimum area of 28m2 in the Urban Living Zone; ii. have a minimum dimension of 6m in the Residential Zone and a minimum dimension of 4m in the Urban Living Zones; and iii. shall be unobstructed by buildings, parking spaces or vehicle access and manoeuvring areas; | Restricted Discretionary | An area of outdoor living space provided on the northern side of the dwelling has an area of (at least) 40m2 and a minimum dimension of 3.8m. The area is unobstructed and directly accessible from the main living area of the dwelling. - As the degree of non-compliance exceeds 10%, the activity status is restricted discretionary. |
| 4.2.10.3 - Building Coverage and Site Permeability | - total impermeable surfaces up to 55% of the land area of the lot or site shall be a Permitted Activity; and - total impermeable surfaces >55% and up to 65% of the land area of the lot or site shall be a Restricted Discretionary Activity (see Criteria in 4.4.11). | Discretionary | The total area of impermeable surfaces, including the dwelling roof area and concrete parking and manoeuvring areas is 283.7m2 which equates to 65.6% of the total site area. |
| 18.2.3.1 - Flooding | All building platforms, other than those for detached and non-habitable Accessory Building must account for flooding and include stormwater system designed in accordance with NZS4404:2010 Land Development and Subdivision Infrastructure Section 4.3.5.2 or subsequent revision, provided that the 13 April 2017 Chapter 18 – Natural Hazards 18-9 minimum free board shall be measured to the building platform level, not the underside of the floor joists or underside of the floor slab. | Discretionary | The minimum building platform level (BPL) for the site is 102.15m (Whakatane Datum). The proposed dwelling will have a timber pile foundation with a BPL of 102.5m which is approximately 1.3m above the current ground level. - To enable practical vehicle access, the attached garage will have a lower platform level of 101.81m (Whakatane Datum) of concrete slab foundation and will not meet the minimum BPL required by Rule 18.2.3.2. |

  - The proposal can comply with all other performance standards.

  - Overall, the proposal has been assessed as a Discretionary Activity.

## National Environmental Standard for Assessing and Managing Contaminants in Soil to Protect Human Health (NESCS)

  - The NESCS seeks to manage actual and potential adverse effects of contamination in soil on human health from particular activities that have occurred on the site. The NESCS includes a Hazardous Activities and Industries List (HAIL) that sets out which activities may have potentially contaminated the soil. The NESCS applies when a person wants to undertake an activity described in subclauses (2) to (6) on a piece of land described in subclause (7) or (8).

  - Although the proposal is a change of use of the site, the application states that the site is not currently being used, has not been used in the past, or is unlikely to have been used for an activity described on the HAIL. The site is not identified by BOPRC as a HAIL site either. Therefore, the NESCS does not apply.

## Other NES

  - There are eight other National Environmental Standards that have been prepared under Sections 43 and 44 of the Act and are in force as regulations. These cover marine aquaculture, fresh water, air quality, human drinking water, plantation forestry, telecommunications facilities, electricity transmission, and storing tyres outdoors.

  - No other NES apply.

# Written approvals

  - The applicant has not consulted with neighbours in respect of this application.

# Assessment of environmental effects

  - As a discretionary activity, all relevant matters can be considered. In this case, the potential adverse effects relate to the risk and effects of a non-compliant outdoor living space, site permeability and inundation. In terms of the later, it is appropriate to expressly consider the matters in Section 3.7.25.

Permitted baseline

  - The District Plan permits construction of a dwelling and garage that fully complies with all the zone standards and other rules.

  - It is not fanciful to anticipate construction of a dwelling on this residential lot. A compliant dwelling would result in visual changes, have temporary construction effects, increase traffic generation, generate additional stormwater and require services. The use, in terms of effects on residential character, are also permitted.

  - In this case, it is appropriate to apply the permitted baseline and to disregard the effects associated with a compliant permitted dwelling. Therefore, the potential adverse effects that must be considered are in relation to the natural hazard of inundation, the non-compliance of the outdoor living space and impermeable surfaces.

Inundation

  - The subject site is not located within an identified flood hazard area in terms of the District Plan’s overlays but modelling held by council has identified that the site is subject to inundation and so a platform level applies. In assessing whether it is appropriate to permit the garage to be constructed below the BPL there are a number of risk factors, which must be considered. These are:

- The risk to property

- The risk to human health

- The risk to Council

- The impact of climate change

Risk to Property

  - The Whakatāne Township is protected from flooding from the Whakatāne River by stopbanks which are designed and maintained to contain a 1% AEP (Annual Exceedance Probability) flood. While these stopbanks do offer protection from the design storm event it must be kept in mind that risks associated with stopbank failure and events that exceed the design capacity still exist. The property is also at risk from localised surface flooding, as illustrated in Figure 7.

  - Council’s Three Waters engineering staff advise that the 1% AEP flood level for the site is 101.65m (Whakatāne Datum). This means a compliant platform level is 102.15m WD when freeboard is added.

  - The proposed attached garage has a level of 101.81m (Whakatāne Datum), of concrete slab design. This will result in the floor being 0.69m below the minimum BPL for an attached garage. Therefore, the attached garage may be subject to inundation. Such an event may result in damage to the content of the garage area and associated property damage.

  - However, the recommended conditions will mitigate and limit the potential risk of damage to the building as they require the use of materials used in the garage to be designed to withstand effects of inundation.

  - The risk of the land being affected by inundation will not increase due to the garage platform level being non-compliant. The dwelling will meet the BPL, so it is only the garage that will suffer damage in the event of a 1% AEP inundation event.

  - On this basis, the effects on property are no more than minor.

- Risk to Life / Human Health

  - In terms of the potential effect on human health, the proposal can comply with the BPL within the dwelling, however the attached non-habitable part of the building does not comply. The potential for negative human health outcomes will not occur provided the garage is maintained as a non-habitable building. A condition is recommended that the garage remain non-habitable. A condition requiring all wiring to be installed at a level equivalent to the minimum BPL  will avoid the risk of electrocution.

- The Risk to Council

  - As the land is subject to inundation, any building consent granted will be issued pursuant to Section 72 of the Building Act. This will highlight to future landowners that the building is being constructed on land which is subject to a natural hazard. Through this process the owners must also acknowledge the identified risk.

- Potential impact of climate change

  - Whakatāne township is generally low lying, and has been built on a historic flood plain, and therefore subject to risks associated with inundation. The chances of flooding are expected to increase as a result of climate change in the coming years.

  - The Whakatāne District Plan Objectives and Policies of Chapter 18 – Natural Hazards require climate change to be considered. This factor has been considered when calculating the AEP level.

  - Overall, the effects of the non-compliance with platform level are less than minor and no other persons or properties are affected by this non-compliance.

Figure 7: Image taken from Council's GIS showing potential ponding levels on the site. Key: Green - <100mm; Amber - 100-300mm; Red - 300mm+.

Outdoor Living Space

  - The outdoor living space does not meet Rule 4.2.7.1(a)(ii), the minimum dimension. The dimension is 3.8m instead of the required 6m.

  - The amenity of the site and the occupants will be not adversely affected by the reduced dimension of the outdoor living space as there is still sufficient space on the site to provide a functional outdoor living area. The design provides for a deck running along the northern side of the dwelling and this is accessible from the main living area and has a total area exceeding 40m2. As all yard setbacks are met, there is also sufficient outdoor recreation area provided for around the dwelling.

  - The outlook from the outdoor living space is unobstructed and does not have views over other properties and is not overlooked, as the site is well fenced and surrounding dwellings are single level.

  - The effect of the non-compliance is less than minor and no other persons or properties are affected by this non-compliance.

Site Permeability

  - Total impermeable surfaces which exceed 55% (and up to 65%) of the land area of the lot are assessed as a Restricted Discretionary Activity, and Council must restrict its discretion to the extent to which the proposal achieves hydrological neutrality and avoids the potential adverse effects of stormwater run-off. Although the activity is full discretionary, these criteria are still useful in terms of assessing effects of this non-compliance.

  - In this case, the building coverage is compliant (32%), but as the dwelling is located on a rear section, the length of the access way must be included in the impermeable area calculations. The total impermeable surface is 65.6%.

  - In this case, compliance with stormwater management also needs to be demonstrated as per the consent notice on the title and this has been demonstrated on the amended site plan Drawing 101 (received 20/10/21). This shows two detention tanks located within the western yard. A condition is recommended to ensure that the tanks are 1m from the boundary, as they are over 1 metre in height, unless written approval is obtained from the adjacent property.

  - Council’s Development Engineer has reviewed the plans and calculations and is satisfied that the additional stormwater can be appropriately managed on the site. A condition is recommended to ensure stormwater management is in accordance with these plans. Therefore, there will not be any adverse ‘off-site’ effects from the non-compliance.

## Conclusion regarding scale of effects

  - Overall, the effects of the proposal on the environment are considered to be less than minor.

  - The effects of the proposal on adjacent properties are nil.  The non-compliant BPL of the attached garage relates to the dwelling and occupants on this site only and will not affect adjoining properties. Stormwater will be managed on site and the non-compliant outdoor living space relates to onsite amenity only.

# Notification assessment and decision

## Public Notification of Consent Applications

  - The steps a consent authority must follow in order to determine whether to publicly notify an application are set out in Section 95A of the RMA.

  - In relation to Step 1, public notification is not mandatory because:

- The applicant has not requested public notification.

- Public notification is not required under Section 95C as all relevant information has been provided as part of the application.

- The application does not involve the exchange of recreation reserve land.

  - In relation to Step 2, public notification is not precluded because:

- The District Plan does not preclude public notification.

- The application is for a discretionary activity but not a boundary activity.

  - In relation to Step 3, public notification is not required because:

- The District Plan does not require public notification.

- The assessment in Section 6 above demonstrates that the adverse effects of the activities on the environment will be less than minor.

- In making the assessment and in considering S95D:

Effects on persons who own the land and adjacent land have been disregarded.

The permitted baseline has been considered as described above.

The activity is not for a restricted discretionary activity so all relevant matters can be considered.

Trade competition effects are not relevant.

  - In relation to Step 4, public notification is not required because:

- There are no special circumstances that exist that require the application to be publicly notified. The application contains all necessary information to determine the application.

  - As such, it is concluded that the application does not need to be publicly notified.

## Potentially Affected Persons

  - The steps a consent authority must follow in order to determine whether to give limited notification of an application are set out in Section 95B of the RMA.

  - In relation to Step 1, no affected groups require notification because:

- The proposal does not affect protected customary rights groups.

- The proposal does not affect customary marine title groups.

- The site is not adjacent to, and will not affect, a statutory acknowledgment area.

  - In relation to Step 2, limited notification is not precluded because:

- The District Plan does not preclude limited notification.

- The application is for a discretionary activity.

  - In relation to Step 3, no specific persons must be notified because:

- The proposal is not a boundary activity.

- The assessment in Section 6 above demonstrates that the adverse effects of the activity on the environment, will be less than minor. The permitted baseline has been taken into account as described above. There is no relevant statutory acknowledgement.

  - In relation to Step 4:

- There are no special circumstances that would warrant limited notification of any other persons.

## Recommendation Regarding Notification

  - Therefore, it is concluded that the application does not require public or limited notification.

## Notification decision

  - Acting under delegated authority and as recommended in Section 6.12 above, resource consent number LL-2020-9257-00 shall be processed on the following basis:

- ☒	Non-notified

- ☐	Limited Notified

- ☐	Notified

| Approved By: |  |
| --- | --- |
|  | Mike Avery Manager Resource Consents |
| Date: | 28 October 2021 |

# Section 104 assessment: Consideration of application

  - When considering an application and any submissions received, the Council must, subject to Part 2, have regard to any actual and potential effects on the environment of allowing the activity.

  - For the reasons set out above, the adverse effects that this proposal generates are than less than minor and are acceptable given the location, zoning and overall design.

## Positive Effects

  - The reduced BPL for the garage facilitates better and more practical access to the building. It also enables provision of a secure parking area.

## Section 104(1)(b)

  - Section 104(1)(b) sets out those documents whose provisions must be considered when determining an application.

## Whakatāne District Plan

  - The proposal is a discretionary activity. The following objectives and policies are relevant to this assessment:

- Chapter 4 - Residential

| Objective Res1 | The maintenance and enhancement of the character and amenity for dwellings and residential activities within Residential and Urban Living Zoned areas. |
| --- | --- |
| Policy 1 | To ensure a site is large enough for a proposed use, development or subdivision to ensure residents on-site or in the neighbourhood retain or will have a high level of residential amenity. |
| Policy 2 | To ensure adequate sunlight and daylight enters dwellings and other forms of residential buildings. |
| Policy 3 | To maintain spatial privacy, usable outdoor space and access about dwellings and other forms of residential buildings. |
| Policy 4 Policy 5 | To maintain and enhance a high level of amenity on residential sites by requiring the provision of; - screened service courts to be provided; - private indoor and outdoor living areas; and - safe on-site parking and maneuvering. To enable changes in land use, including greater density of residential development in urban areas, without compromising the existing level of amenity. |
| The proposal can achieve most of the Residential zone standards and will still provide a high level of residential amenity. The dwelling is appropriately situated on the site to allow adequate access to daylight and maintains privacy between houses and avoids overlooking. Parking and maneuvering can be provided on-site. The design is maximizing useable outdoor space without compromising residential amenity. |  |

- Chapter 18 – Natural Hazards

| Objective Haz1 | Manage the subdivision, use, development and protection of land so as to avoid or mitigate the adverse effects of natural hazards on the life and wellbeing of people, and significant environmental values. |
| --- | --- |
| Policy 4 Policy 11 | To avoid or mitigate the adverse effects of the subdivision, use or development of land that is likely to accelerate, worsen or result in material damage to that land, or other land, or structures, by erosion, falling debris, subsidence, slippage or inundation from any source. To manage the avoidance or mitigation of natural hazards according to their level of risk. |
| Policy 12 | To take into account the effects of climate change when identifying hazards and the locations where those hazards could adversely affect people and property. |
| The dwelling can achieve the minimum BPL required to meet a 1 in 100 year flood event, however the garage does not achieve the required BPL. The garage is a non-habitable space only so there is no risk to life. The building does not in itself change the risk of a hazard affecting the property. Non-compliance with the BPL will be mitigated by conditions so that if the garage flooded in a 1% AEP inundation event flood, damage to the owner/occupier’s property will be reduced. |  |

  - For these reasons, the proposal is considered consistent with the above objectives and policies.

## National and Regional Planning Documents

  - There are no relevant National policy documents that require consideration other than the NPSUD.

- The National Policy Statement on Urban Development (NPSUD 2020):

  - The NPSUD 2020 came into effect on 20 August 2020 and is relevant to assessing the application. Whakatāne District Council is a tier 3 local authority.

| National Policy Statement on Urban Development 2020 |  |
| --- | --- |
| Objective 1 | New Zealand has well-functioning urban environments that enable all people and communities to provide for their social, economic, and cultural wellbeing, and for their health and safety, now and into the future. |
| Objective 4 | New Zealand’s urban environments, including their amenity values, develop and change over time in response to the diverse and changing needs of people, communities, and future generations. |
| Objective 6 | Local authority decisions on urban development that affect urban environments are: - integrated with infrastructure planning and funding decisions; and - strategic over the medium term and long term; and - Responsive, particularly in relation to proposals that would supply significant development capacity. |
| Policy 1 | Planning decisions contribute to well-functioning urban environments, which are urban environments that, as a minimum: - . - . - . - . - . - are resilient to the likely current and future effects of climate change. |
| Policy 6 | When making planning decisions that affect urban environments, decision-makers have particular regard to the following matters: - the planned urban built form anticipated by those RMA planning documents that have given effect to this National Policy Statement - that the planned urban built form in those RMA planning documents may involve significant changes to an area, and those changes: (i) may detract from amenity values appreciated by some people but improve amenity values appreciated by other people, communities, and future generations, including by providing increased and varied housing densities and types; and (ii) are not, of themselves, an adverse effect - the benefits of urban development that are consistent with well-functioning urban environments (as described in Policy 1). - any relevant contribution that will be made to meeting the requirements of this National Policy Statement to provide or realise development capacity. - the likely current and future effects of climate change. |
| Comment: The development of appropriate housing is a matter of significance nationally, regionally and locally. The design of the dwelling reflects the size and constraints of the site and will still provide a suitable degree of on-site amenity. Infill housing provides housing choice and may contribute to affordability and reflects and utilises existing infrastructure. Climate change has been taken into account in terms of the BPL. The dwelling achieves this level, but the non-habitable garage does not. For these reasons and the reasons noted above, the proposal is consistent with the policies of the NPS and achieves its objectives. |  |

- Bay of Plenty Regional Council Regional Policy Statement (BOP RPS):

  - The proposal is not contrary to the relevant provisions of the BOP Regional Policy Statement in relation to the sustainable management of the natural and physical resources of the region.

  - The following objective and policy are relevant to this proposal.

- Objective 31	Avoidance or mitigation of natural hazards by managing risk for people’s safety and the protection of property and lifeline utilities.

- Policy NH 10B: Assessment of natural hazard risk at the time of subdivision, or change or intensification of land use after Policies NH 7A and NH 8A have been given effect to.

  - The above objective and policy direct Council to ensure that risks from natural hazards are assessed and that the effects are mitigated. As discussed above, the proposal will not increase the risk of a flood event or change the level of risk to people’s safety. However, it may increase the damage to property stored in the garage or to the garage structure itself. The owners have accepted this risk and appropriate conditions have been imposed to minimise risk and damage to the extent possible, while still meeting the building code.

## Other Matters and Considerations

  - Section 104(1)(c) provides that, when considering an application for resource consent and any submissions received, the consent authority must, subject to Part 2, have regard to any other matter the consent authority considers relevant and reasonably necessary to determine the application.

  - There are no other relevant documents that need to be considered. The consent does not trigger consultation with Ngāti Awa under their iwi management plan due to the reasons for consent and type of effects being considered.

## Financial Contributions for Roads and Public Reserves

  - A financial contribution was charged with resource consent SS-2019-9001-00 and paid at time of subdivision.

## Development Contributions

  - A development contribution was charged with resource consent SS-2019-9001-00 and paid at time of subdivision.

# Discretionary or non-complying activities Section 104b

  - Section 104B states:

- After considering an application for a resource consent for a discretionary activity or non-complying activity, a consent authority;

- may grant or refuse the application; and

- if it grants the application, may impose conditions under Section 108.

  - I consider the proposed conditions comply with Section 108 of the RMA.

# Part 2 matters

  - Section 6(h) of the RMA lists the management of significant risks from natural hazards. The proposal will not increase the potential of  inundation from occurring on the site. The area of non-compliance is a garage only, which is a non-habitable space. However, it will increase the amount of property which could be damaged in a flood event. Mitigation has been proposed in relation to electrical wiring and the materials to be used in the garage. This is a pragmatic and practical solution given the costs and difficulties in raising the entire garage site.

  - There will be no effect to the quality or amenity of the surrounding environment. The other areas of non-compliance relate to internal effects, being permeability and outdoor living area. The site will provide a functional outdoor living space and all stormwater will be managed onsite or within Council’s reticulation. The overall type and scale of residential development is consistent with the zoning (Section 7).

  - The proposal is not contrary to Te Tiriti o Waitangi (Section 8).

  - Based on the assessment in this report, the proposal is in accordance with the purpose and principles of the RMA, as set out in Part 2. The proposal is a sustainable use of the land. The proposal allows the applicant to construct the proposed dwelling cost effectively, the alternative being that the garage BPL is significantly raised making vehicular access impractical. The health and safety of people and the wider community has been considered, along with economic and social effects. In this case, the non-compliance of the non-habitable attached garage only, presents no risk to life. The applicant accepts that there is a risk from inundation.

# Recommendation

On 28 October 2021, Whakatāne District Council resolved under delegated authority as follows:

- To GRANT a Resource Consent:

- Under section 9(3)(a) of the Resource Management Act 1991 and Rules 18.2.3.2, 4.2.7 and Rule 4.2.10.3 of the Whakatāne District Plan being a discretionary activity and;

- In accordance with sections 34A, 104 and 104B of the Resource Management Act 1991;

To construct a dwelling with an attached garage that has a platform below the minimum building platform level and non-compliant site permeability and outdoor living space on a 469 m2 allotment of land described as:

| Legal description: | LOT 2 DP 548564 |
| --- | --- |
| Record of Title (RT): | 939786 |
| Site Address: | 60a Pohutu Street, Whakatane |

## Subject to the following conditions:

- General

- The development shall be carried out in accordance with the submitted plans stamped “Approved by Planning” (A2117026) and information lodged with the application including the supporting information supplied

- Stormwater

- Stormwater management and maintenance details shall be provided with the building consent application for the dwelling in accordance with the approved plan ‘Drawing Number 101’, dated 27/09/2021 to demonstrate compliance with the consent notice on the title. All maintenance requirements shall be met on an on-going basis.

- Inundation

- The dwelling, excluding the attached garage, shall have a minimum platform level that achieves the 1% AEP level + 500mm freeboard. Refer to advice note (v).

- All electrical wiring and fittings in the garage shall be installed at or above the 1% AEP level + 500mm freeboard level. Refer to advice note (v).

- The garage shall not be used as a habitable room.

- The walls of the garage shall be constructed of materials able to withstand the effects of inundation without experiencing damage that would require their replacement, up to a level of at least the 1% AEP level + 500mm freeboard.  There shall be no insulation or use of gib wall lining below the 1% AEP level + 500mm freeboard. Refer to advice note (v).

- Compliance with conditions 2-6 above shall be demonstrated in plans and specifications included in the Building Consent application for the new dwelling.

- A monitoring fee of $160.00 shall be paid to the Council for the administration, monitoring and supervision of this consent. Notwithstanding the above, where there is good and reasonable cause for unprogrammed monitoring and additional site inspections, then the costs of that will be a charge on the consent holder. Such costs are recovered on an actual and reasonable basis as defined in the General Conditions and Notes of the Fees and Charges Schedule as approved by the Council in terms of Section 36 of the Resource Management Act 1991.

- THAT within 20 working days of the receipt of the invoice, the additional administrative charges that were incurred in relation to the receiving, processing and granting of this resource consent (pursuant to S36 of the Resource Management Act 1991) shall be paid to the Council in full.

## Advice notes:

- If you do not understand any conditions of this consent, please contact the Council’s Planning Department for clarification before starting work.

- Under Section 125 of the RMA, this resource consent will lapse in five years, unless it is given effect to within that time.

- In accordance with Section 127(1) of the RMA, the consent holder may apply to the consent authority for a change or cancellation of any condition of this consent.

- It is the consent holder’s responsibility to comply with any conditions imposed on this resource consents prior to and during (as applicable) exercising this resource consent.

- Under rule 18.2.3.2, the minimum building platform level is currently 102.15 m RL Whakatāne Datum. If there is a delay in giving effect to this consent, this level may change as it is dependent on the results of stormwater modelling and climate change factors.

- In the event that, during earthworks, an archaeological find is made or kōiwi uncovered, work shall stop immediately, the appropriate iwi authority shall be advised and an appropriate course of action shall be determined in accordance with the Heritage New Zealand Pouhere Taonga Act 2014 and the appropriate iwi protocols.

- Building consent will be required for the new dwelling. There is a Section 72 on the record of title as the section is subject to inundation.

- As part of the building consent process a site-specific geotechnical assessment report will be required from a suitably qualified geotechnical engineer to confirm good ground and foundations/slab type.

- Detailed design of services including water, stormwater and wastewater will be required at the time of building consent application.

# Reasons for the decision

  - Section 113(4) of the Resource Management Act 1991 requires that every decision on a resource consent that has not been notified shall be in writing and state reasons for the decision. The reasons for the decision on this resource consent are listed below:

- The actual and potential effects created by the proposal are considered acceptable because:

- The dwelling achieves the minimum building platform level. In the event of a 1% AEP flood, only the garage may be subject to inundation. Conditions have been imposed to ensure all electrical wiring is raised above the minimum building platform level for safety and protection from electrocution. A condition is also proposed in relation to the materials to be used in the garage.

- The garage is a non-habitable building. A condition is included that the garage is to remain non-habitable to avoid the risk to life. The owner/occupier may store their personal belongings in the garage at their own risk.

- In order for the garage to meet the relevant building platform level the applicant would need to increase the driveway incline in an impractical manner.

- Overall, the proposal is a pragmatic solution and will pose minimal risks to inhabitants of the property.

- The dwelling will still have an adequate useable outdoor living area that is accessible from the main living area and north facing.

- All stormwater can be disposed of appropriately in accordance with the consent notice.

- The proposal is considered consistent with the following objectives and policies of the Whakatāne District Plan:

- Objective Res 1 – Policy 1 – 5

- Objective Haz1 – Policy 4, 11, 12

- The proposal is consistent with the relevant matters for discretion in the Whakatāne District Plan and the higher order National and Regional planning documents.

- The applicant has accepted the draft conditions.

- Overall, the proposal meets the purpose (Section 5) and principles (Sections 6-8) of the Resource Management Act 1991 because the proposal is a sustainable use of residential land and all adverse effects can be avoided, remedied and mitigated.  The proposal allows the applicant to construct the proposed dwelling cost effectively, the alternative being that the garage is significantly raised making vehicular access impractical. The health and safety of people has been considered, along with economic and social effects. In this case, the non-compliance of the non-habitable attached garage only, presents no risk to life and the applicant accepts that there is a risk from inundation. The residential use is anticipated by the zoning and adequate on-site amenity is maintained.

| Reporting Planner: |  | Reviewed By: |
| --- | --- | --- |
| Rebecca Hart Environmental Planner |  | Laura Swan Consultant Planner |
| Date: 22 October 2021 |  | Date: 26/10/2021 |

# Decision

  - Acting under delegated authority and as recommended in Section 10 above, resource consent number LL-2020-9257-00 shall be:

- ☐	Granted subject to the conditions of consent specified in Section 10 above.

- ☐	Refused.

| Approved By: |  |
| --- | --- |
|  | Mike Avery Manager Resource Consents |
| Date: | 28 October 2021 |

Ministry for the Environment. “Climate Change Predictions for the Bay of Plenty Region” https://www.mfe.govt.nz/climate-change/likely-impacts-of-climate-change/how-could-climate-change-affect-my-region/bay-of

See Objective Haz1, Policy 12.